EXPERIMENT-4

clc; clear; close all;

%% Part A: Neighbourhoods of a pixel

A = magic(5);

disp('A ='); disp(A);

r = input('Enter row index (2 to 4): ');

c = input('Enter column index (2 to 4): ');

fprintf('Pixel value p(%d,%d) = %d\n', r, c, A(r,c));

N4 = [A(r-1,c), A(r+1,c), A(r,c-1), A(r,c+1)];

ND = [A(r-1,c-1), A(r-1,c+1), A(r+1,c-1), A(r+1,c+1)];

N8 = [N4, ND];

disp('N4(p) ='); disp(N4);

disp('ND(p) ='); disp(ND);

disp('N8(p) ='); disp(N8);

%% Part B: 4- and 8-connectivity

BW = logical([1 1 0 0 0 0;

1 1 0 0 1 0;

0 0 1 0 1 0;

0 0 0 1 0 0;

0 1 0 0 0 1;

0 1 0 0 0 1]);

CC4 = bwconncomp(BW, 4);

CC8 = bwconncomp(BW, 8);

fprintf('Connected components, 4-connectivity: %d\n', CC4.NumObjects);

fprintf('Connected components, 8-connectivity: %d\n', CC8.NumObjects);

%% Part C: Distance between two pixels selected on an image

I = imread('cameraman.tif');

figure; imshow(I); title('Click on two pixels');

[x, y] = ginput(2);

x = round(x); y = round(y);

hold on; plot(x, y, 'r+-', 'MarkerSize', 12, 'LineWidth', 1.5);

De = sqrt((x(1)-x(2))^2 + (y(1)-y(2))^2); % Euclidean

D4 = abs(x(1)-x(2)) + abs(y(1)-y(2)); % city-block

D8 = max(abs(x(1)-x(2)), abs(y(1)-y(2))); % chessboard

fprintf('p = (%d,%d), q = (%d,%d)\n', x(1), y(1), x(2), y(2));

fprintf('Euclidean distance De = %.4f\n', De);

fprintf('City-block distance D4 = %d\n', D4);

fprintf('Chessboard distance D8 = %d\n', D8);

%% Part D: Distance maps from a single pixel

P = false(101); P(51,51) = true;

figure('Name','Distance maps');

subplot(1,3,1); imshow(bwdist(P,'euclidean'), []); title('Euclidean (De)');

subplot(1,3,2); imshow(bwdist(P,'cityblock'), []); title('City-block (D4)');

subplot(1,3,3); imshow(bwdist(P,'chessboard'), []); title('Chessboard (D8)');

RESULT: